ANTENNA Q714E3XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q714E3XL | 13 | 3385 | 16,95 | 30,87 | 20,68 | 19,03 | 28,8 | 29,9 | 18,8 | +4,22 | 5200 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 16,85 | 25,0 | +2,87 |
| Silvered rods | 16,89 | 22,7 | +3,33 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 116 | 205 | 393 | 642 | 916 | 1219 | 1535 | 1846 | 2158 | 2478 | 2775 | 3127 | 3385 |
| Length | 242 | 221 | 215 | 205 | 199 | 191 | 185 | 181 | 178 | 175 | 170 | 165 | 162 | 160 |
| - All elements made of Al rods Ǿ3,2 mm - The height of all rectangles is 135 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q714E3XL | 3385 | No loss | 16,95 | 18,8 | +4,22 | 429,700-434,900 |
|  |  | Aluminum | 16,85 | 25,0 | +2,87 |  |
|  |  | Silver | 16,89 | 22,7 | +3,33 |  |
| EF7014-5 | 3050 | No loss | 16,78 | 22,9 | +3,17 | 430,450-433,650 |
|  |  | Aluminum | 16,70 | 28,4 | +2,16 |  |
|  |  | Silver | 16,73 | 26,4 | +2,52 |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)