ANTENNA Q727D9XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q727D9XL | 27 | 7215 | 19,93 | 28,49 | 22,06 | 21,21 | 20,4 | 20,8 | 15,9 | +7,92 | 4480 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 19,89 | 18,5 | +7,22 |
| Silvered rods | 19,91 | 17,5 | +7,47 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 120 | 198 | 350 | 485 | 690 | 942 | 1222 | 1519 | 1829 | 2139 | 2458 | 2775 | 3096 |
| Length | 284 | 256 | 246 | 223 | 219 | 217 | 213 | 207 | 202 | 198 | 195 | 192 | 189 | 185 |
|  | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 |  |
| Pos | 3427 | 3746 | 4072 | 4403 | 4728 | 5047 | 5369 | 5699 | 6028 | 6327 | 6645 | 6945 | 7215 |  |
| Length | 183 | 181 | 179 | 176 | 174 | 172 | 170 | 170 | 168 | 167 | 165 | 162 | 162 |  |
| - All elements made of Al rods Ǿ10,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q727D9XL | 7215 | No loss | 19,93 | 15,9 | +7,92 | 429,900-434,380 |
|  |  | Aluminum | 19,89 | 18,5 | +7,22 |  |
|  |  | Silver | 19,91 | 17,5 | +7,47 |  |
| EF7032-5 | 8670 | No loss | 19,93 | 19,9 | +6,94 | 429,700-433,980 |
|  |  | Aluminum | 19,85 | 25,5 | +5,78 |  |
|  |  | Silver | 19,88 | 23,3 | +6,20 |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)