ANTENNA Q733D9XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q733D9XL | 33 | 9035 | 20,59 | 47,42 | 21,17 | 20,33 | 19,2 | 19,4 | 15,5 | +8,70 | 5950 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 20,56 | 17,9 | +8,03 |
| Silvered rods | 20,57 | 17,1 | +8,25 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 125 | 204 | 351 | 488 | 688 | 942 | 1222 | 1519 | 1829 | 2142 | 2461 | 2775 |
| Length | 288 | 256 | 245 | 222 | 218 | 217 | 212 | 207 | 201 | 196 | 193 | 191 | 189 |
|  | D12 | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 |
| Pos | 3098 | 3426 | 3750 | 4075 | 4404 | 4723 | 5054 | 5376 | 5705 | 6023 | 6339 | 6644 | 6958 |
| Length | 185 | 182 | 181 | 180 | 175 | 175 | 173 | 171 | 170 | 169 | 168 | 167 | 166 |
|  | D25 | D26 | D27 | D28 | D29 | D30 | D31 | - All elements made of Al rods Ǿ10,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |
| Pos | 7268 | 7551 | 7862 | 8157 | 8486 | 8767 | 9035 |  |  |  |  |  |  |
| Length | 163 | 162 | 160 | 160 | 158 | 158 | 155 |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q733D9XL | 8745 | No loss | 20,59 | 15,5 | +8,70 | 428,850-434,700 |
|  |  | Aluminum | 20,56 | 17,9 | +8,03 |  |
|  |  | Silver | 20,57 | 17,1 | +8,25 |  |
| NO REFERENCE TO COMPARE |  | No loss |  |  |  |  |
|  |  | Aluminum |  |  |  |  |
|  |  | Silver |  |  |  |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)