ANTENNA Q736D9XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q736D9XL | 36 | 9965 | 20,83 | 35,31 | 22,79 | 21,97 | 18,8 | 19,0 | 15,3 | +8,99 | 5300 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 20,80 | 17,7 | +8,32 |
| Silvered rods | 20,81 | 16,7 | +8,58 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 124 | 203 | 353 | 488 | 688 | 942 | 1222 | 1519 | 1829 | 2142 | 2461 | 2775 | 3098 |
| Length | 287 | 256 | 245 | 222 | 218 | 217 | 212 | 207 | 201 | 196 | 193 | 191 | 189 | 185 |
|  | D13 | D14 | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 |
| Pos | 3426 | 3750 | 4075 | 4404 | 4723 | 5054 | 5376 | 5705 | 6023 | 6339 | 6644 | 6958 | 7268 | 7551 |
| Length | 183 | 181 | 179 | 177 | 175 | 173 | 171 | 170 | 169 | 168 | 167 | 166 | 164 | 162 |
|  | D27 | D28 | D29 | D30 | D31 | D32 | D33 | D34 | - All elements made of Al rods Ǿ10,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |
| Pos | 7860 | 8157 | 8486 | 8768 | 9085 | 9395 | 9699 | 9965 |  |  |  |  |  |  |
| Length | 161 | 161 | 159 | 158 | 157 | 156 | 150 | 140 |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q736D9XL | 9965 | No loss | 20,83 | 15,3 | +8,99 | 429,300-434,600 |
|  |  | Aluminum | 20,80 | 17,7 | +8,32 |  |
|  |  | Silver | 20,81 | 16,7 | +8,58 |  |
| NO REFERENCE TO COMPARE |  | No loss |  |  |  |  |
|  |  | Aluminum |  |  |  |  |
|  |  | Silver |  |  |  |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)