ANTENNA Q740D3XL

PERFORMANCES: (No loss condition)

| TYPE | ELE | L (mm) | G (dBi) | F/B (dB) | F/Sh (dBi) | F/Sv (dBi) | Hor (◦) | Ver (◦) | Temp (◦K) | G/T (dB) | ΔF * (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q740D3XL | 40 | 11200 | 21,74 | 40,53 | 18,78 | 18,15 | 16,4 | 16,6 | 15,8 | +9,75 | 6550 |

* ΔF given for SWR=1,5

PERFORMANCES: (Losses included)

| Element material | G (dBi) | Temp (◦K) | G/T (dB) |
| --- | --- | --- | --- |
| Aluminum rods | 21,70 | 18,5 | +9,03 |
| Silvered rods | 21,71 | 17,6 | +9,26 |

DIMENSIONS:

|  | Ref | De | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | D10 | D11 | D12 | D13 | D14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos | 0 | 125 | 202 | 347 | 490 | 687 | 938 | 1223 | 1521 | 1822 | 2129 | 2450 | 2758 | 3093 | 3410 | 3725 |
| Length | 286 | 257 | 244 | 224 | 220 | 217 | 211 | 206 | 202 | 200 | 195 | 193 | 191 | 189 | 187 | 185 |
|  | D15 | D16 | D17 | D18 | D19 | D20 | D21 | D22 | D23 | D24 | D25 | D26 | D27 | D28 | D29 | D30 |
| Pos | 4057 | 4377 | 4702 | 5023 | 5356 | 5666 | 5986 | 6317 | 6622 | 6938 | 7243 | 7536 | 7834 | 8138 | 8446 | 8755 |
| Length | 184 | 183 | 182 | 181 | 180 | 180 | 178 | 176 | 175 | 174 | 173 | 172 | 171 | 170 | 169 | 168 |
|  | D31 | D32 | D33 | D34 | D35 | D36 | D37 | D38 | - All elements made of Al rods Ǿ10,0 mm - The height of all rectangles is 100 mm - All dimensions given in milimetres |  |  |  |  |  |  |  |
| Pos | 9059 | 9389 | 9703 | 10007 | 10339 | 10647 | 10941 | 11200 |  |  |  |  |  |  |  |  |
| Length | 167 | 166 | 165 | 164 | 164 | 164 | 164 | 160 |  |  |  |  |  |  |  |  |

COMPARISON

| Antenna | Boomlength (mm) | Material | Gain (dBi) | TA (◦K) | G/T (dB) | Bandwidth (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Q740D3XL | 11200 | No loss | 21,74 | 15,8 | +9,75 | 428,000-434,550 |
|  |  | Aluminum | 21,70 | 18,5 | +9,03 |  |
|  |  | Silver | 21,71 | 17,6 | +9,26 |  |
| NO REFERENCE TO COMPARE |  |  |  |  |  |  |

DIAGRAMS

NO LOSS INCLUDED

LOSSES INCLUDED (aluminum)

LOSSES INCLUDED (silver)