| ANTENNA QY72505XL2D13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  | 11 |  |  |  | 13 |  |  |  | 25 |  |  | 6225 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 | D5 |  | D6 | D7 |  | D8 | D9 | D10 |  |
| Position | 0 | 55 |  | 105 | 137 | 239 |  | 432 | 661 | 914 |  | 1186 | 1458 |  | 1748 | 2043 | 2343 |  |
| Length | 336 | 345 |  | 292 | 311 | 311 |  | 301 | 295 | 289.5 |  | 286.5 | 283 |  | 279.5 | 276.5 | 274 |  |
|  | DQ1 | DQ2 |  | DQ3 | DQ4 | DQ5 |  | DQ6 | DQ7 | DQ8 |  | DQ9 | DQ10 |  | DQ11 | DQ12 | DQ13 |  |
| Position | 2632 | 2932 |  | 3221 | 3519 | 3821 |  | 4132 | 4444 | 4754 |  | 5051 | 5377 |  | 5652 | 5944 | 6225 |  |
| Length | 159 | 157 |  | 155 | 153 | 150 |  | 146 | 142 | 142 |  | 140 | 140 |  | 140 | 131 | 131 |  |

| Remark | All elements made of Al tubes Ǿ5 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.28 |  | 29.77 |  | 19.72 | 19.27 |  | 22.2 |  |  | 22.8 | 23.8 |  |  | +6.49 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.19 | 6.3 |  | 23.9 |  |  | +5.21 | 426.500 – 437.200 |  |  |  |
| After KF2YN correction |  |  |  | 19.11 | 6.3 |  | 29.6 |  |  | +4.40 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2040 |  |  |  | 2040 |  |  |  |
| Vertical distance |  |  |  | 1880 |  |  |  |  |  |  | 1880 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1880 |  |  | 22.06 |  |  | 29.2 |  | 6.3 |  |  |  | +7.41 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2040 |  |  | 22.11 |  |  | 29.4 |  |  | 6.3 |  |  |  | +7.43 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2040 |  |  | 25.10 |  |  | 29.1 |  |  | 6.3 |  |  |  | +10.46 |  |
| V = 1880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |