| ANTENNA QY72506XL1D13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  | 11 |  |  |  | 13 |  |  |  | 25 |  |  | 6225 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 | D5 |  | D6 | D7 |  | D8 | D9 | D10 |  |
| Position | 0 | 55 |  | 105 | 138 | 243 |  | 434 | 658 | 911 |  | 1184 | 1456 |  | 1746 | 2045 | 2342 |  |
| Length | 334 | 342 |  | 296 | 304 | 307 |  | 297.5 | 291 | 285 |  | 283 | 278.5 |  | 275 | 272.5 | 268.5 |  |
|  | DQ1 | DQ2 |  | DQ3 | DQ4 | DQ5 |  | DQ6 | DQ7 | DQ8 |  | DQ9 | DQ10 |  | DQ11 | DQ12 | DQ13 |  |
| Position | 2635 | 2932 |  | 3221 | 3519 | 3818 |  | 4138 | 4436 | 4752 |  | 5052 | 5354 |  | 5662 | 5954 | 6225 |  |
| Length | 159 | 157 |  | 155 | 151 | 147 |  | 145 | 141 | 141 |  | 141 | 141 |  | 133 | 133 | 133 |  |

| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.09 |  | 30.22 |  | 19.74 | 19.56 |  | 22.6 |  |  | 23.2 | 24.0 |  |  | +6.27 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.19 | 4.6 |  | 24.7 |  |  | +5.29 | 423.500 – 438.500 |  |  |  |
| After KF2YN correction |  |  |  | 19.12 | 4.6 |  | 28.9 |  |  | +4.51 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1920 |  |  |  | 1920 |  |  |  |
| Vertical distance |  |  |  | 1880 |  |  |  |  |  |  | 1880 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1880 |  |  | 22.07 |  |  | 28.2 |  | 4.6 |  |  |  | +7.57 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1920 |  |  | 22.07 |  |  | 28.6 |  |  | 4.6 |  |  |  | +7.51 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1920 |  |  | 25.07 |  |  | 28.1 |  |  | 4.6 |  |  |  | +10.58 |  |
| V = 1880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |