| ANTENNA QY72510XL8D18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |  |  |
| LFA |  |  | 6 |  |  |  | 18 |  |  |  | 25 |  |  | 6225 |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 | D5 |  | DQ1 | DQ2 |  | DQ3 | DQ4 | DQ5 |  |
| Position | 0 | 70 |  | 120 | 152 | 245 |  | 435 | 672 | 932 |  | 1203 | 1478 |  | 1775 | 2086 | 2378 |  |
| Length | 331 | 327 |  | 327 | 295 | 301.5 |  | 289.5 | 281.5 | 274.5 |  | 171 | 163 |  | 157 | 157 | 155 |  |
|  | DQ6 | DQ7 |  | DQ8 | DQ9 | DQ10 |  | DQ11 | DQ12 | DQ13 |  | DQ14 | DQ15 |  | DQ16 | DQ17 | DQ18 |  |
| Position | 2678 | 2972 |  | 3262 | 3556 | 3852 |  | 4148 | 4454 | 4752 |  | 5068 | 5356 |  | 5666 | 5972 | 6225 |  |
| Length | 153 | 151 |  | 149 | 147 | 145 |  | 143 | 141 | 139 |  | 137 | 135 |  | 131 | 127 | 123 |  |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.29 |  | 31.65 |  | 20.22 | 17.58 |  | 22.2 |  |  | 22.4 | 24.0 |  |  | +6.11 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.24 | 3.2 |  | 24.1 |  |  | +5.47 | 426.500 – 437.700 |  |  |  |
| After KF2YN correction |  |  |  | 19.19 | 3.2 |  | 26.9 |  |  | +4.89 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 1980 |  |  |  | 1980 |  |  |  |
| Vertical distance |  |  |  | 1940 |  |  |  |  |  |  | 1940 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1940 |  |  | 22.17 |  |  | 26.3 |  | 3.2 |  |  |  | +7.97 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 1980 |  |  | 22.18 |  |  | 26.6 |  |  | 3.2 |  |  |  | +7.93 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 1980 |  |  | 25.19 |  |  | 26.1 |  |  | 3.2 |  |  |  | +11.02 |  |
| V = 1940 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |