| ANTENNA QY72608XL1D19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  |  | Boomlength (mm) |  |  |  |
| LFA |  |  | 6 |  |  |  | 19 |  |  |  | 26 |  |  |  | 6525 |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 | D5 |  | DQ1 | DQ2 | DQ3 |  | DQ4 | DQ5 | DQ6 |
| Position | 0 | 60 |  | 110 | 148 | 245 |  | 431 | 659 | 919 |  | 1188 | 1472 | 1754 |  | 2056 | 2352 | 2638 |
| Length | 330 | 335 |  | 335 | 307 | 306 |  | 296 | 288 | 279 |  | 172 | 165 | 161 |  | 157 | 157 | 155 |
|  | DQ7 | DQ8 |  | DQ9 | DQ10 | DQ11 |  | DQ12 | DQ13 | DQ14 |  | DQ15 | DQ16 | DQ17 |  | DQ18 | DQ19 |  |
| Position | 2926 | 3212 |  | 3508 | 3795 | 4108 |  | 4402 | 4706 | 5022 |  | 5337 | 5646 | 5935 |  | 6268 | 6525 |  |
| Length | 153 | 151 |  | 150 | 149 | 147 |  | 145 | 143 | 143 |  | 141 | 137 | 133 |  | 133 | 131 |  |

| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.52 |  | 31.33 |  | 19.39 | 17.27 |  | 21.4 |  |  | 21.6 | 24.2 |  |  | +6.33 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.46 | 4.6 |  | 24.2 |  |  | +5.44 | 426.100 – 436.800 |  |  |  |
| After KF2YN correction |  |  |  | 19.40 | 4.6 |  | 28.4 |  |  | +4.87 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2480 |  |  |  | 2480 |  |  |  |
| Vertical distance |  |  |  | 1960 |  |  |  |  |  |  | 1960 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 1960 |  |  | 22.37 |  |  | 27.8 |  | 4.6 |  |  |  | +7.93 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2480 |  |  | 22.47 |  |  | 28.2 |  |  | 4.6 |  |  |  | +7.97 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2480 |  |  | 25.39 |  |  | 27.7 |  |  | 4.6 |  |  |  | +10.96 |  |
| V = 1960 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |