| ANTENNA QY72805XL6D16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 11 |  |  | 16 |  |  | 28 |  |  | 7125 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 |  | D4 | D5 |  | D6 | D7 |
| Position | 0 | 55 |  | 105 | 137 |  | 239 | 432 |  | 660 | 912 |  | 1186 | 1458 |
| Length | 335 | 341 |  | 304 | 310.5 |  | 311 | 301 |  | 295 | 289.5 |  | 287 | 283.5 |
|  | D8 | D9 |  | D10 | DQ1 |  | DQ2 | DQ3 |  | DQ4 | DQ5 |  | DQ6 | DQ7 |
| Position | 1747 | 2039 |  | 2339 | 2631 |  | 2932 | 3221 |  | 3519 | 3821 |  | 4137 | 4444 |
| Length | 279.5 | 276.5 |  | 273 | 161 |  | 157 | 156 |  | 155 | 151 |  | 149 | 147 |
|  | DQ8 | DQ9 |  | DQ10 | DQ11 |  | DQ12 | DQ13 |  | DQ14 | DQ15 |  | DQ16 |  |
| Position | 4749 | 5051 |  | 5362 | 5654 |  | 5958 | 6268 |  | 6572 | 6886 |  | 7125 |  |
| Length | 145 | 145 |  | 143 | 139 |  | 135 | 131 |  | 129 | 129 |  | 129 |  |

| Remark | All elements made of Al tubes Ǿ5 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 19.74 |  | 38.85 |  | 19.41 | 18.46 |  | 21.0 |  |  | 21.4 | 23.2 |  |  | +7.12 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 19.65 | 6.3 |  | 23.3 |  |  | +5.77 | 425.000 – 436.800 |  |  |  |
| After KF2YN correction |  |  |  | 19.57 | 6.3 |  | 29.0 |  |  | +4.95 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2580 |  |  |  | 2580 |  |  |  |
| Vertical distance |  |  |  | 2020 |  |  |  |  |  |  | 2020 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2020 |  |  | 22.54 |  |  | 28.8 |  | 6.3 |  |  |  | +7.95 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2580 |  |  | 22.64 |  |  | 29.0 |  |  | 6.3 |  |  |  | +8.02 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2580 |  |  | 25.58 |  |  | 28.8 |  |  | 6.3 |  |  |  | +10.99 |  |
| V = 2020 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |