| ANTENNA QY72908XL7D22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 22 |  |  | 29 |  |  | 7315 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 |  | D4 | D5 |  | DQ1 | DQ2 |
| Position | 0 | 55 |  | 111 | 148 |  | 240 | 425 |  | 653 | 905 |  | 1168 | 1452 |
| Length | 331 | 335 |  | 335 | 306 |  | 306 | 294 |  | 288 | 281 |  | 173 | 164 |
|  | DQ3 | DQ4 |  | DQ5 | DQ6 |  | DQ7 | DQ8 |  | DQ9 | DQ10 |  | DQ11 | DQ12 |
| Position | 1738 | 2035 |  | 2326 | 2615 |  | 2914 | 3193 |  | 3488 | 3782 |  | 4085 | 4367 |
| Length | 161 | 159 |  | 158 | 156 |  | 155 | 151 |  | 149 | 149 |  | 148 | 148 |
|  | DQ13 | DQ14 |  | DQ15 | DQ16 |  | DQ17 | DQ18 |  | DQ19 | DQ20 |  | DQ21 | DQ22 |
| Position | 4665 | 4955 |  | 5252 | 5540 |  | 5857 | 6142 |  | 6454 | 6746 |  | 7054 | 7315 |
| Length | 147 | 145 |  | 145 | 145 |  | 141 | 141 |  | 139 | 139 |  | 139 | 133 |

| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 20.09 |  | 32.06 |  | 18.58 | 16.83 |  | 19.8 |  |  | 20.0 | 25.1 |  |  | +6.81 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 20.03 | 4.3 |  | 25.1 |  |  | +5.98 | 424.800 – 437.800 |  |  |  |
| After KF2YN correction |  |  |  | 19.96 | 4.3 |  | 29.0 |  |  | +5.34 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2480 |  |  |  | 2480 |  |  |  |
| Vertical distance |  |  |  | 2080 |  |  |  |  |  |  | 2080 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2080 |  |  | 22.92 |  |  | 28.3 |  | 4.3 |  |  |  | +8.40 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2480 |  |  | 23.03 |  |  | 28.9 |  |  | 4.3 |  |  |  | +8.42 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2480 |  |  | 26.00 |  |  | 28.1 |  |  | 4.3 |  |  |  | +11.51 |  |
| V = 2080 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |