| ANTENNA QY73106XL3D24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 24 |  |  |  | 31 |  |  | 7935 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 | D4 |  | D5 | DQ1 |  | DQ2 | DQ3 |
| Position | 0 | 50 |  | 105 | 144 |  | 244 | 429 | 665 |  | 920 | 1185 |  | 1466 | 1758 |
| Length | 334 | 355 |  | 288 | 309.5 |  | 308.5 | 298 | 291 |  | 284.5 | 173 |  | 163 | 161 |
|  | DQ4 | DQ5 |  | DQ6 | DQ7 |  | DQ8 | DQ9 | DQ10 |  | DQ11 | DQ12 |  | DQ13 | DQ14 |
| Position | 2052 | 2344 |  | 2628 | 2924 |  | 3208 | 3502 | 3799 |  | 4098 | 4382 |  | 4688 | 4966 |
| Length | 159 | 158 |  | 156 | 155 |  | 153 | 150 | 150 |  | 150 | 150 |  | 150 | 150 |
|  | DQ15 | DQ16 |  | DQ17 | DQ18 |  | DQ19 | DQ20 | DQ21 |  | DQ22 | DQ23 |  | DQ24 |  |
| Position | 5259 | 5562 |  | 5850 | 6158 |  | 6456 | 6762 | 7064 |  | 7366 | 7668 |  | 7935 |  |
| Length | 145 | 145 |  | 145 | 143 |  | 143 | 141 | 141 |  | 135 | 133 |  | 133 |  |

| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 20.34 |  | 34.87 |  | 17.05 | 15.58 |  | 19.4 |  |  | 19.4 | 24.4 |  |  | +7.21 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 20.27 | 5.2 |  | 24.4 |  |  | +6.21 | 424.900 – 438.300 |  |  |  |
| After KF2YN correction |  |  |  | 20.20 | 5.2 |  | 29.1 |  |  | +5.56 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2560 |  |  |  | 2560 |  |  |  |
| Vertical distance |  |  |  | 2060 |  |  |  |  |  |  | 2060 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2060 |  |  | 23.10 |  |  | 28.5 |  | 5.2 |  |  |  | +8.55 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2560 |  |  | 23.28 |  |  | 28.8 |  |  | 5.2 |  |  |  | +8.68 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2560 |  |  | 26.20 |  |  | 28.1 |  |  | 5.2 |  |  |  | +11.71 |  |
| V = 2060 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |