| ANTENNA QY73208XL4D25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  | Number of oblong elements |  |  |  | Total number of elements |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  | 25 |  |  |  | 32 |  |  | 8235 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 |  | D2 | D3 | D4 |  | D5 | DQ1 |  | DQ2 | DQ3 |
| Position | 0 | 70 |  | 120 | 158 |  | 258 | 443 | 672 |  | 928 | 1201 |  | 1476 | 1765 |
| Length | 332 | 300 |  | 300 | 303.5 |  | 305 | 295.5 | 288 |  | 279.5 | 173 |  | 165 | 162 |
|  | DQ4 | DQ5 |  | DQ6 | DQ7 |  | DQ8 | DQ9 | DQ10 |  | DQ11 | DQ12 |  | DQ13 | DQ14 |
| Position | 2073 | 2363 |  | 2661 | 2946 |  | 3232 | 3516 | 3806 |  | 4081 | 4374 |  | 4649 | 4943 |
| Length | 159 | 157 |  | 156 | 155 |  | 154 | 153 | 152 |  | 151 | 150 |  | 149 | 148 |
|  | DQ15 | DQ16 |  | DQ17 | DQ18 |  | DQ19 | DQ20 | DQ21 |  | DQ22 | DQ23 |  | DQ24 | DQ25 |
| Position | 5233 | 5512 |  | 5824 | 6126 |  | 6432 | 6731 | 7047 |  | 7374 | 7671 |  | 7976 | 8235 |
| Length | 147 | 146 |  | 145 | 144 |  | 143 | 142 | 141 |  | 141 | 139 |  | 137 | 137 |

| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 20.63 |  | 29.14 |  | 16.89 | 15.77 |  | 18.6 |  |  | 18.6 | 25.1 |  |  | +7.21 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 20.56 | 4.6 |  | 25.2 |  |  | +6.38 | 425.000 – 437.300 |  |  |  |
| After KF2YN correction |  |  |  | 20.51 | 4.6 |  | 29.3 |  |  | +5.84 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2580 |  |  |  | 2580 |  |  |  |
| Vertical distance |  |  |  | 2560 |  |  |  |  |  |  | 2560 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2560 |  |  | 23.57 |  |  | 29.5 |  | 4.6 |  |  |  | +8.87 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2580 |  |  | 23.57 |  |  | 28.8 |  |  | 4.6 |  |  |  | +8.98 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2580 |  |  | 26.62 |  |  | 29.3 |  |  | 4.6 |  |  |  | +11.95 |  |
| V = 2560 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |