| ANTENNA QY73410XL2D27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  | Total number of elements |  |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  |  | 27 |  |  | 34 |  |  |  | 8835 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 |  | D5 | DQ1 | DQ2 |  | DQ3 | DQ4 |
| Position | 0 | 56 |  | 102 | 142 | 243 |  | 428 | 652 |  | 909 | 1175 | 1444 |  | 1743 | 2038 |
| Length | 330 | 331 |  | 331 | 298 | 300 |  | 290.5 | 282.5 |  | 275.5 | 172 | 165 |  | 159 | 156 |
|  | DQ5 | DQ6 |  | DQ7 | DQ8 | DQ9 |  | DQ10 | DQ11 |  | DQ12 | DQ13 | DQ14 |  | DQ15 | DQ16 |
| Position | 2342 | 2648 |  | 2929 | 3234 | 3507 |  | 3804 | 4078 |  | 4361 | 4666 | 4957 |  | 5266 | 5559 |
| Length | 153 | 152 |  | 151 | 147 | 143 |  | 143 | 143 |  | 143 | 141 | 139 |  | 139 | 139 |
|  | DQ17 | DQ18 |  | DQ19 | DQ20 | DQ21 |  | DQ22 | DQ23 |  | DQ24 | DQ25 | DQ26 |  | DQ27 |  |
| Position | 5866 | 6166 |  | 6463 | 6771 | 7056 |  | 7372 | 7686 |  | 7982 | 8284 | 8582 |  | 8835 |  |
| Length | 139 | 139 |  | 133 | 133 | 131 |  | 131 | 131 |  | 127 | 127 | 123 |  | 121 |  |

| Remark | All elements made of Al tubes Ǿ10 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 20.45 |  | 38.00 |  | 19.10 | 18.02 |  | 19.4 |  |  | 19.6 | 23.9 |  |  | +7.11 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 20.41 | 3.5 |  | 23.9 |  |  | +6.48 | 426.000 – 439.000 |  |  |  |
| After KF2YN correction |  |  |  | 20.36 | 3.5 |  | 27.1 |  |  | +6.04 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2380 |  |  |  | 2380 |  |  |  |
| Vertical distance |  |  |  | 2380 |  |  |  |  |  |  | 2380 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2380 |  |  | 23.38 |  |  | 27.0 |  | 3.5 |  |  |  | +9.07 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2380 |  |  | 23.38 |  |  | 27.0 |  |  | 3.5 |  |  |  | +9.07 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2380 |  |  | 26.44 |  |  | 26.7 |  |  | 3.5 |  |  |  | +12.16 |  |
| V = 2380 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |