| ANTENNA QY73508XL8D28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  | Number of yagi elements |  |  |  | Number of oblong elements |  |  | Total number of elements |  |  |  | Boomlength (mm) |  |  |
| LFA |  |  | 6 |  |  |  | 28 |  |  | 35 |  |  |  | 9125 |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref | De1 |  | De2 | D1 | D2 |  | D3 | D4 |  | D5 | DQ1 | DQ2 |  | DQ3 | DQ4 |
| Position | 0 | 70 |  | 120 | 158 | 259 |  | 443 | 672 |  | 928 | 1201 | 1476 |  | 1765 | 2073 |
| Length | 332 | 330 |  | 330 | 303.5 | 305 |  | 295.5 | 288 |  | 280 | 173 | 165 |  | 162 | 159 |
|  | DQ5 | DQ6 |  | DQ7 | DQ8 | DQ9 |  | DQ10 | DQ11 |  | DQ12 | DQ13 | DQ14 |  | DQ15 | DQ16 |
| Position | 2363 | 2661 |  | 2946 | 3232 | 3516 |  | 3806 | 4081 |  | 4374 | 4649 | 4943 |  | 5233 | 5512 |
| Length | 157 | 156 |  | 155 | 154 | 153 |  | 152 | 151 |  | 150 | 149 | 148 |  | 147 | 146 |
|  | DQ17 | DQ18 |  | DQ19 | DQ20 | DQ21 |  | DQ22 | DQ23 |  | DQ24 | DQ25 | DQ26 |  | DQ27 | DQ28 |
| Position | 5824 | 6126 |  | 6432 | 6731 | 7047 |  | 7374 | 7671 |  | 7986 | 8283 | 8591 |  | 8883 | 9125 |
| Length | 145 | 144 |  | 143 | 142 | 141 |  | 141 | 139 |  | 137 | 137 | 137 |  | 137 | 133 |

| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  | F/Sh (dBi) | F/Sv (dBi) |  | Hor (◦) |  |  | Ver (◦) | Temp (◦K) |  |  | G/T (dB) |
| 20.98 |  | 34.15 |  | 17.89 | 16.61 |  | 18.0 |  |  | 18.2 | 24.8 |  |  | +7.56 |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  | G (dBi) | Tloss (K) |  | TA (◦K) |  |  | G/T (dB) | Bandwidth for SWR=1.5 (MHz) |  |  |  |
| After NEC/EZNEC |  |  |  | 20.92 | 4.9 |  | 24.9 |  |  | +6.71 | 425.400 – 437.750 |  |  |  |
| After KF2YN correction |  |  |  | 20.86 | 4.9 |  | 29.3 |  |  | +6.20 |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Two antennas vertically (mm) |  |  | Two antennas horizontally (mm) |  |  |  | Four antennas in H-stack (mm) |  |  |  |
| Horizontal distance |  |  |  |  |  |  | 2640 |  |  |  | 2640 |  |  |  |
| Vertical distance |  |  |  | 2420 |  |  |  |  |  |  | 2420 |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  | Tloss (K) |  |  |  | G/T (dB) |  |  |
| 2420 |  |  | 23.87 |  |  | 29.2 |  | 4.9 |  |  |  | +9.22 |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| 2640 |  |  | 23.92 |  |  | 28.8 |  |  | 4.9 |  |  |  | +9.33 |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  | G (dBi) |  |  | TA (K) |  |  | Tloss (K) |  |  |  | G/T (dB) |  |
| H = 2640 |  |  | 26.93 |  |  | 29.1 |  |  | 4.9 |  |  |  | +12.30 |  |
| V = 2420 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |