| ANTENNA QY73805XL3D31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  |  | Number of yagi elements |  |  |  |  |  | Number of oblong elements |  |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |  |
| LFA |  |  |  |  | 6 |  |  |  |  |  | 31 |  |  |  |  |  | 38 |  |  |  |  |  |  | 10025 |  |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 |  | D1 | D2 |  |  |  | D3 |  | D4 | D5 |  |  |  |  | DQ1 | DQ2 |  |  | DQ3 |  |  | DQ4 |  | DQ5 |
| Position | 0 |  | 50 |  |  | 100 |  | 132 | 233 |  |  |  | 426 |  | 656 | 918 |  |  |  |  | 1190 | 1463 |  |  | 1764 |  |  | 2055 |  | 2352 |
| Length | 336 |  | 349 |  |  | 290 |  | 309 | 310 |  |  |  | 300 |  | 293.5 | 287 |  |  |  |  | 171 | 163 |  |  | 160 |  |  | 158 |  | 157 |
|  | DQ6 |  | DQ7 |  |  | DQ8 |  | DQ9 | DQ10 |  |  |  | DQ11 |  | DQ12 | DQ13 |  |  |  |  | DQ14 | DQ15 |  |  | DQ16 |  |  | DQ17 |  | DQ18 |
| Position | 2632 |  | 2924 |  |  | 3208 |  | 3502 | 3799 |  |  |  | 4098 |  | 4382 | 4688 |  |  |  |  | 4966 | 5262 |  |  | 5558 |  |  | 5846 |  | 6162 |
| Length | 156 |  | 154 |  |  | 153 |  | 150 | 150 |  |  |  | 150 |  | 150 | 150 |  |  |  |  | 150 | 150 |  |  | 150 |  |  | 145 |  | 145 |
|  | DQ19 |  | DQ20 |  |  | DQ21 |  | DQ22 | DQ23 |  |  |  | DQ24 |  | DQ25 | DQ26 |  |  |  |  | DQ27 | DQ28 |  |  | DQ29 |  |  | DQ30 |  | DQ31 |
| Position | 6460 |  | 6754 |  |  | 7062 |  | 7366 | 7678 |  |  |  | 7974 |  | 8282 | 8578 |  |  |  |  | 8876 | 9178 |  |  | 9476 |  |  | 9768 |  | 10025 |
| Length | 145 |  | 145 |  |  | 141 |  | 135 | 135 |  |  |  | 133 |  | 133 | 133 |  |  |  |  | 133 | 133 |  |  | 133 |  |  | 131 |  | 131 |
| Remark | All elements made of Al tubes Ǿ5 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  |  | G/T (dB) |  |
| 21.02 |  | 34.30 |  |  |  |  | 17.38 |  |  | 16.15 |  |  |  | 18.2 |  |  |  |  |  | 18.4 |  |  | 24.3 |  |  |  |  |  | +7.96 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  |  | 20.94 |  |  | 5.7 |  |  |  | 24.4 |  |  |  |  |  | +6.83 |  |  | 424.500 – 438.100 |  |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  |  | 20.87 |  |  | 5.7 |  |  |  | 29.5 |  |  |  |  |  | +6.17 |  |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  |  | 2660 |  |  |  |  |  |  |  |  | 2660 |  |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  |  | 2880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2880 |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 2880 |  |  |  | 23.95 |  |  |  |  |  |  |  | 29.4 |  |  |  |  |  | 5.7 |  |  |  |  |  |  |  | +9.26 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 2660 |  |  |  | 23.92 |  |  |  |  |  |  |  | 29.2 |  |  |  |  |  |  | 5.7 |  |  |  |  |  |  |  | +9.26 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 2660 |  |  |  | 26.99 |  |  |  |  |  |  |  | 29.1 |  |  |  |  |  |  | 5.7 |  |  |  |  |  |  |  | +12.34 |  |  |  |
| V = 2880 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |