| ANTENNA QY73806XL4D31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  |  | Number of yagi elements |  |  |  |  |  | Number of oblong elements |  |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |  |
| LFA |  |  |  |  | 6 |  |  |  |  |  | 31 |  |  |  |  |  | 38 |  |  |  |  |  |  | 10025 |  |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 |  | D1 | D2 |  |  |  | D3 |  | D4 | D5 |  |  |  |  | DQ1 | DQ2 |  |  | DQ3 |  |  | DQ4 |  | DQ5 |
| Position | 0 |  | 60 |  |  | 105 |  | 143 | 244 |  |  |  | 427 |  | 659 | 917 |  |  |  |  | 1190 | 1467 |  |  | 1756 |  |  | 2062 |  | 2352 |
| Length | 333 |  | 332 |  |  | 332 |  | 303.5 | 307.5 |  |  |  | 298 |  | 291 | 284 |  |  |  |  | 171 | 165 |  |  | 162 |  |  | 159 |  | 157 |
|  | DQ6 |  | DQ7 |  |  | DQ8 |  | DQ9 | DQ10 |  |  |  | DQ11 |  | DQ12 | DQ13 |  |  |  |  | DQ14 | DQ15 |  |  | DQ16 |  |  | DQ17 |  | DQ18 |
| Position | 2649 |  | 2935 |  |  | 3221 |  | 3507 | 3796 |  |  |  | 4072 |  | 4362 | 4638 |  |  |  |  | 4930 | 5225 |  |  | 5503 |  |  | 5815 |  | 6114 |
| Length | 156 |  | 154 |  |  | 153 |  | 150 | 150 |  |  |  | 150 |  | 150 | 150 |  |  |  |  | 150 | 150 |  |  | 150 |  |  | 145 |  | 145 |
|  | DQ19 |  | DQ20 |  |  | DQ21 |  | DQ22 | DQ23 |  |  |  | DQ24 |  | DQ25 | DQ26 |  |  |  |  | DQ27 | DQ28 |  |  | DQ29 |  |  | DQ30 |  | DQ31 |
| Position | 6421 |  | 6719 |  |  | 7039 |  | 7361 | 7657 |  |  |  | 7971 |  | 8274 | 8585 |  |  |  |  | 8884 | 9172 |  |  | 9474 |  |  | 9768 |  | 10025 |
| Length | 143 |  | 142 |  |  | 141 |  | 140 | 139 |  |  |  | 138 |  | 137 | 137 |  |  |  |  | 137 | 137 |  |  | 133 |  |  | 131 |  | 131 |
| Remark | All elements made of Al tubes Ǿ6 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  |  | G/T (dB) |  |
| 21.19 |  | 31.08 |  |  |  |  | 17.75 |  |  | 16.58 |  |  |  | 17.8 |  |  |  |  |  | 18.0 |  |  | 25.0 |  |  |  |  |  | +7.74 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  |  | 21.12 |  |  | 4.9 |  |  |  | 25.0 |  |  |  |  |  | +6.86 |  |  | 424.500 – 438.700 |  |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  |  | 21.07 |  |  | 4.9 |  |  |  | 28.4 |  |  |  |  |  | +6.54 |  |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  |  | 2760 |  |  |  |  |  |  |  |  | 2760 |  |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  |  | 2620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2620 |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 2620 |  |  |  | 24.10 |  |  |  |  |  |  |  | 29.8 |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +9.36 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 2760 |  |  |  | 24.12 |  |  |  |  |  |  |  | 29.2 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +9.47 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 2760 |  |  |  | 27.15 |  |  |  |  |  |  |  | 29.6 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +12.44 |  |  |  |
| V = 2620 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |