| ANTENNA QY73808XL3D31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  |  | Number of yagi elements |  |  |  |  |  | Number of oblong elements |  |  |  |  |  | Total number of elements |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |  |
| LFA |  |  |  |  | 6 |  |  |  |  |  | 31 |  |  |  |  |  | 38 |  |  |  |  |  |  | 10025 |  |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 |  | D1 | D2 |  |  |  | D3 |  | D4 | D5 |  |  |  |  | DQ1 | DQ2 |  |  | DQ3 |  |  | DQ4 |  | DQ5 |
| Position | 0 |  | 70 |  |  | 120 |  | 158 | 257 |  |  |  | 443 |  | 672 | 928 |  |  |  |  | 1201 | 1476 |  |  | 1765 |  |  | 2073 |  | 2363 |
| Length | 332 |  | 331 |  |  | 331 |  | 303.5 | 305 |  |  |  | 295 |  | 288 | 280 |  |  |  |  | 173 | 165 |  |  | 162 |  |  | 159 |  | 157 |
|  | DQ6 |  | DQ7 |  |  | DQ8 |  | DQ9 | DQ10 |  |  |  | DQ11 |  | DQ12 | DQ13 |  |  |  |  | DQ14 | DQ15 |  |  | DQ16 |  |  | DQ17 |  | DQ18 |
| Position | 2661 |  | 2946 |  |  | 3232 |  | 3516 | 3806 |  |  |  | 4081 |  | 4374 | 4649 |  |  |  |  | 4943 | 5233 |  |  | 5512 |  |  | 5824 |  | 6126 |
| Length | 156 |  | 155 |  |  | 154 |  | 153 | 152 |  |  |  | 151 |  | 150 | 149 |  |  |  |  | 148 | 147 |  |  | 146 |  |  | 145 |  | 145 |
|  | DQ19 |  | DQ20 |  |  | DQ21 |  | DQ22 | DQ23 |  |  |  | DQ24 |  | DQ25 | DQ26 |  |  |  |  | DQ27 | DQ28 |  |  | DQ29 |  |  | DQ30 |  | DQ31 |
| Position | 6432 |  | 6731 |  |  | 7047 |  | 7374 | 7671 |  |  |  | 7986 |  | 8283 | 8591 |  |  |  |  | 8886 | 9172 |  |  | 9478 |  |  | 9766 |  | 10025 |
| Length | 143 |  | 143 |  |  | 141 |  | 141 | 139 |  |  |  | 139 |  | 139 | 139 |  |  |  |  | 139 | 131 |  |  | 131 |  |  | 131 |  | 125 |
| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  | Hor (◦) |  |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  |  | G/T (dB) |  |
| 21.32 |  | 29.98 |  |  |  |  | 17.22 |  |  | 16.23 |  |  |  | 17.4 |  |  |  |  |  | 17.4 |  |  | 24.3 |  |  |  |  |  | +8.06 |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  | TA (◦K) |  |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  |  | 21.25 |  |  | 4.9 |  |  |  | 24.3 |  |  |  |  |  | +7.17 |  |  | 424.400 – 437.200 |  |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  |  | 21.12 |  |  | 4.9 |  |  |  | 28.8 |  |  |  |  |  | +6.54 |  |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  |  | 2780 |  |  |  |  |  |  |  |  | 2780 |  |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  |  | 2840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2840 |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |
| 2840 |  |  |  | 24.25 |  |  |  |  |  |  |  | 28.9 |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +9.65 |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| 2780 |  |  |  | 24.24 |  |  |  |  |  |  |  | 28.5 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +9.70 |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |
| H = 2780 |  |  |  | 27.29 |  |  |  |  |  |  |  | 28.6 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +12.74 |  |  |  |
| V = 2840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |