| ANTENNA QY74108XL2D34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radiator |  |  |  |  | Number of yagi elements |  |  |  |  |  | Number of oblong elements |  |  |  |  |  | Total number of elements |  |  |  |  |  |  |  | Boomlength (mm) |  |  |  |  |  |  |  |
| LFA |  |  |  |  | 6 |  |  |  |  |  | 34 |  |  |  |  |  | 41 |  |  |  |  |  |  |  | 10925 |  |  |  |  |  |  |  |
| DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ref |  | De1 |  |  | De2 |  | D1 | D2 |  |  |  | D3 | D4 |  | D5 |  |  |  |  | DQ1 | DQ2 |  | DQ3 |  |  |  | DQ4 |  | DQ5 | DQ6 |  |
| Position | 0 |  | 70 |  |  | 120 |  | 158 | 257 |  |  |  | 443 | 672 |  | 928 |  |  |  |  | 1201 | 1476 |  | 1765 |  |  |  | 2073 |  | 2363 | 2661 |  |
| Length | 332 |  | 331 |  |  | 331 |  | 303.5 | 305 |  |  |  | 295 | 288 |  | 280 |  |  |  |  | 173 | 165 |  | 162 |  |  |  | 159 |  | 157 | 156 |  |
|  | DQ7 |  | DQ8 |  |  | DQ9 |  | DQ10 | DQ11 |  |  |  | DQ12 | DQ13 |  | DQ14 |  |  |  |  | DQ15 | DQ16 |  | DQ17 |  |  |  | DQ18 |  | DQ19 | DQ20 |  |
| Position | 2946 |  | 3232 |  |  | 3516 |  | 3806 | 4081 |  |  |  | 4374 | 4649 |  | 4943 |  |  |  |  | 5233 | 5512 |  | 5824 |  |  |  | 6126 |  | 6432 | 6731 |  |
| Length | 155 |  | 154 |  |  | 153 |  | 152 | 151 |  |  |  | 150 | 149 |  | 148 |  |  |  |  | 147 | 146 |  | 145 |  |  |  | 144 |  | 143 | 142 |  |
|  | DQ21 |  | DQ22 |  |  | DQ23 |  | DQ24 | DQ25 |  |  |  | DQ26 | DQ27 |  | DQ28 |  |  |  |  | DQ29 | DQ30 |  | DQ31 |  |  |  | DQ32 |  | DQ33 | DQ34 |  |
| Position | 7047 |  | 7374 |  |  | 7671 |  | 7986 | 8283 |  |  |  | 8591 | 8886 |  | 9172 |  |  |  |  | 9478 | 9776 |  | 10072 |  |  |  | 10368 |  | 10672 | 10925 |  |
| Length | 141 |  | 141 |  |  | 139 |  | 137 | 137 |  |  |  | 137 | 137 |  | 131 |  |  |  |  | 131 | 131 |  | 131 |  |  |  | 131 |  | 125 | 125 |  |
| Remark | All elements made of Al tubes Ǿ8 mm The height of rectangles is 170 mm All dimensions given in milimetres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (No loss condition) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G (dBi) |  | F/B (dB) |  |  |  |  | F/Sh (dBi) |  |  | F/Sv (dBi) |  |  |  |  | Hor (◦) |  |  |  |  | Ver (◦) |  |  | Temp (◦K) |  |  |  |  |  | G/T (dB) |  |  |  |
| 21.59 |  | 34.11 |  |  |  |  | 17.60 |  |  | 16.83 |  |  |  |  | 16.8 |  |  |  |  | 16.8 |  |  | 23.9 |  |  |  |  |  | +8.42 |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and verticall pattern in free space, no loss condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCES (Loss included for aluminum) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  |  |  |  |  |  | G (dBi) |  |  | Tloss (K) |  |  |  |  | TA (◦K) |  |  |  |  | G/T (dB) |  |  | Bandwidth for SWR=1.5 (MHz) |  |  |  |  |  |  |  |  |  |
| After NEC/EZNEC |  |  |  |  |  |  | 21.52 |  |  | 4.9 |  |  |  |  | 23.9 |  |  |  |  | +7.51 |  |  | 424.700 – 437.100 |  |  |  |  |  |  |  |  |  |
| After KF2YN correction |  |  |  |  |  |  | 21.46 |  |  | 4.9 |  |  |  |  | 28.4 |  |  |  |  | +6.94 |  |  |  |  |  |  |  |  |  |  |  |  |
| Remark: Red coloured data are the final antenna data, which means: KF2YN correction formula applied and loss for aluminum included |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIAGRAMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency diagram for SWR better than 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWR diagram for the range from 432.000 – 435.000 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal and vertical pattern in free space, loss included for aluminum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STACKING (ALL DATA GIVEN IN ACCORDANCE WITH KF2YN CORRECTION, LOSS INCLUDED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED STACKING DISTANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Two antennas vertically (mm) |  |  |  |  |  |  |  | Two antennas horizontally (mm) |  |  |  |  |  |  |  | Four antennas in H-stack (mm) |  |  |  |  |  |  |  |  |  |
| Horizontal distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2880 |  |  |  |  |  |  |  | 2880 |  |  |  |  |  |  |  |  |  |
| Vertical distance |  |  |  |  |  |  | 2820 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2820 |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED VERTICALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |  |  |
| 2820 |  |  |  | 24.50 |  |  |  |  |  |  |  | 28.5 |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +9.96 |  |  |  |  |  |  |
| Two antennas stacked vertically |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TWO ANTENNAS STACKED HORIZONTALLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |  |
| 2880 |  |  |  | 24.51 |  |  |  |  |  |  |  | 28.3 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +10.00 |  |  |  |  |  |
| Two antennas stacked horizontally |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOUR ANTENNAS IN H-STACK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCE (mm) |  |  |  | G (dBi) |  |  |  |  |  |  |  | TA (K) |  |  |  |  |  |  | Tloss (K) |  |  |  |  |  |  |  | G/T (dB) |  |  |  |  |  |
| H = 2880 |  |  |  | 27.55 |  |  |  |  |  |  |  | 28.4 |  |  |  |  |  |  | 4.9 |  |  |  |  |  |  |  | +13.03 |  |  |  |  |  |
| V = 2820 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Four antennas in H-stack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |